Bicycle Counter v2.0

Full Project Specification

Target Hardware: NVIDIA Jetson Orin Nano (8 GB)

Firmware: JetPack 6.x / L4T 39.2.0

CUDA: 13.2 | TensorRT: bundled | Python: 3.12

Camera: Suyin HD USB Camera (/dev/video0, 640x480 @ 15 fps)

Database: MySQL 8.x

Document version: 2.0

Date: July 2026

# Table of Contents

1. Project Overview

2. System Architecture

3. Configuration System

4. File-by-File Reference

5. Data Flow - Step by Step

6. Database Schema

7. Dashboard API Reference

8. Deployment & Running

9. Systemd Service

10. Backup & Maintenance

11. Troubleshooting

12. Build from Scratch

13. Possible Upgrades

Appendix: Quick Reference Card

# 1. Project Overview

A real-time bicycle counting system deployed on an NVIDIA Jetson Orin Nano. A USB camera captures video frames; a YOLO11n TensorRT model detects bicycles at ~10-18 ms per frame; a custom IoU+centroid tracker ensures each bicycle is counted exactly once; when a tracked bicycle crosses a horizontal line drawn across the middle of the frame, the system records the crossing event (timestamp, track ID) into a MySQL database and serves all analytics through a live web dashboard at port 8080.

## Features

| Feature | Detail |
| --- | --- |
| Object detection | YOLO11n nano (80 COCO classes, only class 1 = bicycle counted) |
| Inference | TensorRT FP16 engine on Orin GPU, ~10-18 ms/frame |
| Tracking | IoU + centroid distance tracker with persistent IDs across frames |
| Counting | Objects crossing horizontal line Y=240 are counted |
| Deduplication | Spatial + temporal dedup prevents counting the same bike twice |
| Screenshots | JPEG saved on every crossing event to screenshots/ |
| Database | MySQL with per-device, per-location events and 15-min aggregated counts |
| Web dashboard | FastAPI at port 8080: live MJPEG video, stat cards, charts, events |
| Database viewer | Separate page with full history, CSV export, auto-refresh |
| Health endpoint | JSON health check at /health for monitoring |
| Config-driven | All parameters in config.yaml with BIKE_* env var overrides |
| Multi-camera ready | Each camera identified by device_name + location_name |

# 2. System Architecture

## Architecture Diagram

Jetson Orin Nano

|

+------------------+     +----------------------------------------+

| USB Camera       |---->|  bicycle_counter.py (main loop)         |

| /dev/video0      |     |                                        |

| 640x480@15fps    |     |  1. cv2.VideoCapture(0).read()         |

+------------------+     |  2. model(frame) -> detections          |

|  3. tracker.update(raw_dets)            |

|  4. check persist + dedup + count       |

|  5. save screenshot + db.record()       |

|  6. draw_osd() -> JPEG encode           |

|  7. state.latest_jpeg = jpeg_bytes      |

+----------------+------------------------+

|

+----------------v------------------------+

|  dashboard_server.py (FastAPI)           |

|  Port 8080 (0.0.0.0)                    |

|  GET /        -> dashboard.html          |

|  GET /video   -> MJPEG stream            |

|  GET /stats   -> JSON snapshot           |

|  GET /health  -> Health check            |

|  GET /database -> DB viewer              |

|  GET /api/*   -> JSON data               |

|  GET /export/* -> CSV downloads          |

+----------------+------------------------+

|

+----------------v------------------------+

|  database.py (MySQL)                     |

|  Connection pool (size=4)                |

|  Tables: devices, locations,             |

|          crossings, quarterly_counts     |

|  screenshots/ (max 1000 files)           |

+----------------------------------------+

|

+----------------------------------------+

|  config.yaml  <-  config_loader.py      |

|                   BIKE_* env vars       |

+----------------------------------------+

## Key Design Decisions

| Decision | Rationale |
| --- | --- |
| TensorRT over PyTorch | Orin Nano GPU (compute capability 8.7) does not match published PyTorch wheels; ultralytics loads TensorRT engine directly |
| Custom tracker over DeepStream NvDCF | DeepStream 8.0 no longer provides Python pyds bindings for tracking data; lightweight IoU+centroid tracker is sufficient |
| MySQL over SQLite | Production database with connection pooling; supports multi-device deployments and concurrent dashboard reads |
| Horizontal line counting | Simplest approach for counting objects moving across a scene; line at Y=240 (center of 480px frame) |
| Deduplication | Spatial + temporal dedup prevents counting the same bicycle if it lingers near the counting line |
| Config-driven | All parameters in config.yaml with environment variable overrides; no code changes needed for tuning |
| PIL for OSD | Pillow used for drawing bounding boxes and text overlays (avoids OpenCV font limitations) |

# 3. Configuration System

## How it works

config_loader.py reads config.yaml from the project directory

Every parameter can be overridden by an environment variable prefixed with BIKE_

Type coercion is automatic (bool, int, float, str based on the default value)

The flattened config dict is passed to all modules at startup

## config.yaml structure

version: "2.0"

device:

name: "jetson-orin-01"          # Unique device identifier

description: "Jetson Orin Nano - Main Unit"

location_name: "Camera_01"      # Camera location name

location_description: "Main entrance"

camera:

device: "/dev/video0"           # V4L2 device path

width: 640                      # Capture width

height: 480                     # Capture height

fps: 15                         # Target frame rate

brightness: -30                 # V4L2 brightness (-64 to 64)

model:

path: "yolo11n.engine"          # TensorRT engine path

bicycle_class_id: 1             # COCO class ID for bicycle

min_confidence: 0.45            # Minimum detection confidence

tracker:

missing_ttl: 3.0                # Seconds before invisible track is dropped

persist_time: 0.6               # Seconds a track must exist before counting

iou_threshold: 0.15             # Minimum IoU for matching

max_distance: 150               # Max centroid distance (px) for fallback

dedup:

ttl: 5.0                        # Seconds to remember counted positions

distance: 120                   # Min distance (px) between counted positions

database:

host: "127.0.0.1"

port: 3306

user: "bikecounter"

password: "BikeCount2026!"

name: "bicycle_counter"

pool_size: 4                    # MySQL connection pool size

retention:

max_screenshots: 1000           # Max screenshots on disk

keep_screenshots: 950           # Keep N newest after cleanup

max_crossing_days: 90           # Delete crossings older than N days

dashboard:

host: "0.0.0.0"                # Listen address

port: 8080                      # Listen port

logging:

level: "INFO"                   # DEBUG, INFO, WARNING, ERROR

file: "bicycle_counter.log"     # Log file path

## Environment Variable Overrides

Every config key can be overridden. Examples:

export BIKE_CAMERA_FPS=30         # Override camera.fps

export BIKE_MIN_CONFIDENCE=0.6    # Override model.min_confidence

export BIKE_DB_HOST=192.168.1.100 # Override database.host

export BIKE_DASH_PORT=9090        # Override dashboard.port

export BIKE_LOG_LEVEL=DEBUG       # Override logging.level

Full mapping (YAML key -> env var -> default):

| YAML Path | Env Var | Default |
| --- | --- | --- |
| device.name | BIKE_DEVICE_NAME | jetson-orin-01 |
| device.description | BIKE_DEVICE_DESC | "" |
| device.location_name | BIKE_LOCATION_NAME | Camera_01 |
| device.location_description | BIKE_LOCATION_DESC | "" |
| camera.device | BIKE_CAMERA_DEVICE | /dev/video0 |
| camera.width | BIKE_CAMERA_WIDTH | 640 |
| camera.height | BIKE_CAMERA_HEIGHT | 480 |
| camera.fps | BIKE_CAMERA_FPS | 15 |
| camera.brightness | BIKE_CAMERA_BRIGHTNESS | -30 |
| model.path | BIKE_MODEL_PATH | yolo11n.engine |
| model.bicycle_class_id | BIKE_CLASS_ID | 1 |
| model.min_confidence | BIKE_MIN_CONFIDENCE | 0.45 |
| tracker.missing_ttl | BIKE_TRACK_MISSING_TTL | 3.0 |
| tracker.persist_time | BIKE_TRACK_PERSIST_TIME | 0.6 |
| tracker.iou_threshold | BIKE_TRACK_IOU | 0.15 |
| tracker.max_distance | BIKE_TRACK_MAX_DIST | 150 |
| dedup.ttl | BIKE_DEDUP_TTL | 5.0 |
| dedup.distance | BIKE_DEDUP_DIST | 120 |
| database.host | BIKE_DB_HOST | 127.0.0.1 |
| database.port | BIKE_DB_PORT | 3306 |
| database.user | BIKE_DB_USER | bikecounter |
| database.password | BIKE_DB_PASS | BikeCount2026! |
| database.name | BIKE_DB_NAME | bicycle_counter |
| database.pool_size | BIKE_DB_POOL | 4 |
| retention.max_screenshots | BIKE_MAX_SCREENSHOTS | 1000 |
| retention.keep_screenshots | BIKE_KEEP_SCREENSHOTS | 950 |
| retention.max_crossing_days | BIKE_MAX_CROSSING_DAYS | 90 |
| dashboard.host | BIKE_DASH_HOST | 0.0.0.0 |
| dashboard.port | BIKE_DASH_PORT | 8080 |
| logging.level | BIKE_LOG_LEVEL | INFO |
| logging.file | BIKE_LOG_FILE | bicycle_counter.log |

# 4. File-by-File Reference

## Core Python

| File | Lines | Purpose |
| --- | --- | --- |
| bicycle_counter.py | 326 | Main pipeline. Loads config, YOLO model, camera, MySQL, dashboard. Runs main loop: capture -> infer -> track -> count -> screenshot -> OSD -> JPEG encode. Handles signals, camera reconnect, graceful shutdown. |
| dashboard_server.py | 271 | FastAPI web server. 10 endpoints (HTML, MJPEG, JSON, CSV). Runs in daemon thread. Serves dashboard, video stream, stats, health check, database viewer, CSV exports. |
| database.py | 210 | MySQL module. Connection pooling (size 4). CRUD for devices, locations, crossings, quarterly_counts. Atomic upserts for 15-min aggregation. Data retention + screenshot cleanup. |
| config_loader.py | 75 | Config loader. Reads config.yaml, flattens to dict, applies BIKE_* env var overrides with type coercion. |
| export_csv.py | 18 | Standalone CSV export. Connects to MySQL and dumps both quarterly counts and detailed crossings to CSV files. |

## Shell Scripts

| File | Lines | Purpose |
| --- | --- | --- |
| run.sh | 32 | Launcher. Sets LD_LIBRARY_PATH/GST_PLUGIN_PATH for DeepStream/CUDA. Kills camera-holding processes. Prints dashboard URLs. Executes bicycle_counter.py. |
| setup.sh | 403 | Full installer (7 steps). System packages, Python deps, DeepStream-Yolo parser build, YOLO11n download/ONNX export, TensorRT engine build, verification. |
| backup.sh | 32 | Backup script. Dumps MySQL via mysqldump, tars screenshots, compresses both, cleans up backups older than 30 days. |

## Configuration & Service

| File | Lines | Purpose |
| --- | --- | --- |
| config.yaml | 66 | Central configuration. All tunable parameters in one YAML file. |
| bike-counter.service | 21 | Systemd unit file. Runs as system service, depends on mysql.service, auto-restarts on failure. |

## Templates (HTML)

| File | Lines | Purpose |
| --- | --- | --- |
| templates/dashboard.html | 177 | Main dashboard. Dark-themed, mobile-first. Live MJPEG feed, stat cards, crossing timeline, 15-min bar chart, recent detections. Polls /stats every 500ms. Chart.js 4. |
| templates/database.html | 104 | Database viewer. Summary stats, 15-min counts table, all events table, CSV downloads. Auto-refreshes every 5 seconds. |

## Model Files

| File | Size | Purpose |
| --- | --- | --- |
| yolo11n.engine | 7.9 MB | Pre-built TensorRT FP16 engine. Must be rebuilt if moving to a different GPU. |
| yolo11n.onnx | 10.7 MB | ONNX export of YOLO11n. Intermediate for TensorRT engine build. |
| yolo11n.pt | 5.4 MB | Original PyTorch weights. Used for ONNX export only. |

## Reference Files (not used in current pipeline)

| File | Purpose |
| --- | --- |
| config_infer_primary.txt | DeepStream nvinfer config (reference only) |
| deepstream_app_config.txt | DeepStream app config (reference only) |
| labels.txt | COCO 80-class label list |
| libnvdsinfer_custom_impl_Yolo.so | Compiled DeepStream-Yolo parser (reference only) |

## Data (auto-generated)

| Path | Purpose |
| --- | --- |
| screenshots/*.jpg | Crossing event screenshots, named bike_HH-MM-SS_idN.jpg |
| bicycle_counter.log | Application log file |
| bicycle_counts.csv | Exported 15-minute aggregated counts |
| bicycle_crossings_detail.csv | Exported individual crossing events |
| backups/ | Database dumps and screenshot tarballs |

# 5. Data Flow - Step by Step

## 5.1 Startup Sequence

run.sh is executed:

Sets LD_LIBRARY_PATH to include DeepStream and CUDA 13.2

Sets GST_PLUGIN_PATH for GStreamer plugins

Sets PIP_BREAK_SYSTEM_PACKAGES=1

Runs fuser -k /dev/video0 to kill any camera-holding process

Waits 0.5s for camera release

Prints dashboard URLs (localhost + LAN IP)

Executes bicycle_counter.py via exec

bicycle_counter.py initializes:

Loads config from config.yaml via config_loader.load()

Sets up logging (console + file)

Creates screenshots/ directory if missing

Loads YOLO11n TensorRT engine: YOLO("yolo11n.engine")

Connects to MySQL, registers device and location (get-or-create)

Runs cleanup_old_data() to enforce retention policy

Opens USB camera via OpenCV V4L2 backend

Sets camera brightness via v4l2-ctl

Starts dashboard_server in a daemon thread

Enters main processing loop

## 5.2 Main Loop (per frame)

capture frame (cv2.VideoCapture.read)

|

v

YOLO inference (model(frame, conf=0.45))

|

v

Filter: keep only class 1 (bicycle)

|

v

Tracker.update(raw_detections)

- Expire old tracks (not seen > 3.0s)

- Match existing tracks to new detections (IoU >= 0.15, then centroid <= 150px)

- Create new tracks for unmatched detections

|

v

For each tracked detection:

- If track is new (< 0.6s old), skip

- If already counted, skip

- Check dedup: is there a counted position within 120px in last 5.0s?

- If not duplicate: mark counted, increment total, record screenshot + DB

|

v

Draw OSD (bounding boxes + count) using Pillow

|

v

Encode JPEG (quality=75) -> state.latest_jpeg

|

v

Save screenshot for new bikes (quality=90)

|

v

Record crossing in MySQL (insert into crossings + upsert quarterly_counts)

|

v

Cleanup screenshots if > 1000 files

## 5.3 Camera Reconnect

After 30 consecutive cap.read() failures:

Release current camera handle

Wait 1 second

Try open_camera() again

If still fails, wait 5 seconds and retry

## 5.4 Graceful Shutdown

SIGTERM and SIGINT (Ctrl+C) trigger _shutdown event

Main loop exits, prints final count

Camera is released, DB connection closed

# 6. Database Schema

## MySQL Tables

### devices

CREATE TABLE devices (

id          INT AUTO_INCREMENT PRIMARY KEY,

name        VARCHAR(100) NOT NULL UNIQUE,

description TEXT,

created_at  TIMESTAMP DEFAULT CURRENT_TIMESTAMP

);

### locations

CREATE TABLE locations (

id          INT AUTO_INCREMENT PRIMARY KEY,

device_id   INT NOT NULL,

name        VARCHAR(100) NOT NULL,

description TEXT,

created_at  TIMESTAMP DEFAULT CURRENT_TIMESTAMP,

FOREIGN KEY (device_id) REFERENCES devices(id),

UNIQUE KEY (device_id, name)

);

### crossings

CREATE TABLE crossings (

id          INT AUTO_INCREMENT PRIMARY KEY,

device_id   INT NOT NULL,

location_id INT NOT NULL,

recorded_at DATETIME NOT NULL,

track_id    INT NOT NULL,

created_at  TIMESTAMP DEFAULT CURRENT_TIMESTAMP,

FOREIGN KEY (device_id) REFERENCES devices(id),

FOREIGN KEY (location_id) REFERENCES locations(id),

INDEX idx_recorded (recorded_at),

INDEX idx_location (location_id, device_id)

);

### quarterly_counts

CREATE TABLE quarterly_counts (

id            INT AUTO_INCREMENT PRIMARY KEY,

device_id     INT NOT NULL,

location_id   INT NOT NULL,

bucket_date   DATE NOT NULL,

bucket_hour   TINYINT NOT NULL,

bucket_quarter TINYINT NOT NULL,  -- 0,1,2,3 (each = 15 min)

total_count   INT DEFAULT 1,

updated_at    TIMESTAMP DEFAULT CURRENT_TIMESTAMP ON UPDATE CURRENT_TIMESTAMP,

FOREIGN KEY (device_id) REFERENCES devices(id),

FOREIGN KEY (location_id) REFERENCES locations(id),

UNIQUE KEY (device_id, location_id, bucket_date, bucket_hour, bucket_quarter)

);

## Quarterly Bucket Logic

The 15-minute bucket is calculated as: minute // 15

| Time Range | bucket_hour | bucket_quarter | Display |
| --- | --- | --- | --- |
| 00:00 - 00:14 | 0 | 0 | 00:00 |
| 00:15 - 00:29 | 0 | 1 | 00:15 |
| 00:30 - 00:44 | 0 | 2 | 00:30 |
| 00:45 - 00:59 | 0 | 3 | 00:45 |
| 13:30 - 13:44 | 13 | 2 | 13:30 |

## Atomic Upsert

When recording a crossing, the quarterly count is updated atomically:

INSERT INTO quarterly_counts

(device_id, location_id, bucket_date, bucket_hour, bucket_quarter, total_count)

VALUES (%s, %s, %s, %s, %s, 1)

ON DUPLICATE KEY UPDATE total_count = total_count + 1

This ensures no race conditions between the main pipeline and dashboard reads.

# 7. Dashboard API Reference

## Endpoints

| Method | Path | Content-Type | Description |
| --- | --- | --- | --- |
| GET | / | text/html | Main dashboard (renders dashboard.html via Jinja2) |
| GET | /video | multipart/x-mixed-replace | MJPEG stream of latest camera frame with OSD |
| GET | /stats | application/json | Full stats snapshot |
| GET | /health | application/json | Health check JSON |
| GET | /database | text/html | Database viewer page |
| GET | /api/crossings | application/json | All crossing events for this device/location |
| GET | /api/quarterly | application/json | All 15-min aggregated counts |
| GET | /export/csv | text/csv | Download quarterly counts as CSV |
| GET | /export/detail | text/csv | Download all individual crossings as CSV |

## /stats Response

{

"bicycle_count": 42,

"fps": 14.5,

"frame_count": 12345,

"active_tracks": 2,

"total_detections": 150,

"camera_ok": true,

"today_total": 42,

"all_time_total": 91,

"quarterly": [

{"label": "13:30", "count": 32},

{"label": "13:45", "count": 39}

],

"events": [

{"time_str": "13:42:15", "track_id": "7"},

{"time_str": "13:41:02", "track_id": "6"}

]

}

## /health Response

{

"status": "healthy",

"version": "2.0",

"uptime_seconds": 3600.0,

"camera_ok": true,

"database_ok": true,

"bicycle_count": 42,

"device_name": "jetson-orin-01",

"location_name": "Camera_01"

}

Status is "healthy" when both camera and database are OK, "degraded" otherwise.

## /api/crossings Response

[

{

"id": 1,

"timestamp": "2026-07-17T13:42:15",

"track_id": 7,

"location": "Camera_01"

}

]

## /api/quarterly Response

[

{

"date": "2026-07-17",

"period": "13:30-13:45",

"count": 32

}

]

## CORS

All origins are allowed. Only GET methods are permitted. This is read-only access.

# 8. Deployment & Running

## Quick Start

cd ~/CounterProject

./run.sh

## What run.sh Does

Sets LD_LIBRARY_PATH for DeepStream and CUDA 13.2 libraries

Sets GST_PLUGIN_PATH for GStreamer plugins

Runs fuser -k /dev/video0 to kill any camera-holding process

Waits 0.5s for camera release

Prints dashboard URLs

Executes bicycle_counter.py via exec

## Accessing the Dashboard

| URL | When to use |
| --- | --- |
| http://localhost:8080 | On the Jetson itself |
| http://<JETSON_IP>:8080 | From any device on the same LAN |
| http://localhost:8080/health | Health check for monitoring |
| http://localhost:8080/database | Database viewer |

## Stopping the System

Press Ctrl+C in the terminal where run.sh is running. The final bicycle count is printed to the log.

## Running Without run.sh (Manual)

export LD_LIBRARY_PATH="/opt/nvidia/deepstream/deepstream/lib:/usr/local/cuda-13.2/lib64:${LD_LIBRARY_PATH:-}"

export GST_PLUGIN_PATH="/opt/nvidia/deepstream/deepstream/lib/gst-plugins:${GST_PLUGIN_PATH:-}"

fuser -k /dev/video0 2>/dev/null; sleep 0.5

python3 ~/CounterProject/bicycle_counter.py

## MySQL Setup (First Time)

sudo mysql -u root -e "CREATE DATABASE IF NOT EXISTS bicycle_counter;"

sudo mysql -u root -e "CREATE USER IF NOT EXISTS 'bikecounter'@'127.0.0.1' IDENTIFIED BY 'BikeCount2026!';"

sudo mysql -u root -e "GRANT ALL PRIVILEGES ON bicycle_counter.* TO 'bikecounter'@'127.0.0.1';"

sudo mysql -u root -e "FLUSH PRIVILEGES;"

Tables are created automatically on first run.

# 9. Systemd Service

## Service File: bike-counter.service

[Unit]

Description=Bicycle Counter Pipeline

After=network.target mysql.service

Wants=mysql.service

[Service]

Type=simple

User=student

WorkingDirectory=/home/student/CounterProject

ExecStartPre=/usr/bin/fuser -k /dev/video0 2>/dev/null || true

ExecStart=/home/student/CounterProject/run.sh

Restart=on-failure

RestartSec=5

TimeoutStopSec=10

KillMode=mixed

KillSignal=SIGTERM

Environment=LD_LIBRARY_PATH=/opt/nvidia/deepstream/deepstream/lib:/usr/local/cuda-13.2/lib64

Environment=PIP_BREAK_SYSTEM_PACKAGES=1

[Install]

WantedBy=multi-user.target

## Installing and Enabling

sudo cp bike-counter.service /etc/systemd/system/

sudo systemctl daemon-reload

sudo systemctl enable bike-counter.service

sudo systemctl start bike-counter.service

## Managing the Service

sudo systemctl status bike-counter.service    # Check status

sudo systemctl stop bike-counter.service      # Stop

sudo systemctl restart bike-counter.service   # Restart

sudo journalctl -u bike-counter -f            # Follow logs

sudo journalctl -u bike-counter --since today  # Today's logs

## Disabling the Service

sudo systemctl stop bike-counter.service

sudo systemctl disable bike-counter.service

# 10. Backup & Maintenance

## Backup Script: backup.sh

./backup.sh                    # Default: ./backups/

./backup.sh /path/to/backups   # Custom backup directory

What it does:

Dumps MySQL database via mysqldump (compressed with gzip)

Creates tarball of screenshots/ directory

Deletes backup files older than 30 days

## Data Retention

Enforced automatically at startup and on each crossing event:

| What | Limit | Behavior |
| --- | --- | --- |
| Screenshots | Max 1000 files | Deletes oldest, keeps 950 newest |
| Crossing events | Max 90 days | Deletes rows older than 90 days |
| Quarterly counts | Max 90 days | Deletes rows older than 90 days |

## Log Rotation

For production, add a logrotate config:

# /etc/logrotate.d/bicycle-counter

/home/student/CounterProject/bicycle_counter.log {

daily

rotate 7

compress

missingok

notifempty

}

## Database Credentials

Stored in config.yaml under database: section. Default credentials:

User: bikecounter

Password: BikeCount2026!

Database: bicycle_counter

Host: 127.0.0.1:3306

IMPORTANT: Change these credentials for production deployments.

# 11. Troubleshooting

## Camera Issues

| Symptom | Fix |
| --- | --- |
| Cannot open camera / cap.isOpened() False | Run fuser -k /dev/video0 then restart |
| Camera overexposed (white in sun) | v4l2-ctl -d /dev/video0 --set-ctrl brightness=-40 (range: -64 to 64) |
| VIDEOIO ERROR: V4L2 pixel format not supported | Camera does not support MJPEG at 640x480. Try different USB camera. |
| Low frame rate | Check camera.fps in config.yaml (default 15) |
| Camera reconnects repeatedly | Check USB cable, power supply, or try different USB port |

## Inference Issues

| Symptom | Fix |
| --- | --- |
| model(frame) hangs or takes > 1s | TensorRT engine may be built for different GPU. Rebuild: trtexec --onnx=yolo11n.onnx --saveEngine=yolo11n.engine --fp16 |
| CUDA out-of-memory | Orin Nano shares 8GB between CPU/GPU. Close other GPU apps. |
| ultralytics not found | pip3 install ultralytics with export PIP_BREAK_SYSTEM_PACKAGES=1 |
| PyTorch CUDA mismatch warnings | Normal - ultralytics bypasses PyTorch for TensorRT. Harmless. |

## Database Issues

| Symptom | Fix |
| --- | --- |
| Access denied for user bikecounter | Check MySQL is running: sudo systemctl status mysql. Verify credentials in config.yaml. |
| Can not connect to MySQL server | Ensure MySQL is running: sudo systemctl start mysql. Check host/port in config. |
| Connection pool exhausted | Increase database.pool_size in config.yaml (default: 4). |
| Want to reset all data | sudo mysql -u root -e "DROP DATABASE bicycle_counter; CREATE DATABASE bicycle_counter;" |
| Want to reset screenshots | rm ~/CounterProject/screenshots/*.jpg |

## Dashboard Issues

| Symptom | Fix |
| --- | --- |
| Port 8080 already in use | fuser -k 8080/tcp then restart |
| Dashboard loads but video is black | Pipeline crashed. Check terminal for [ERROR] messages. |
| Dashboard shows Pipeline not running | Main loop not running. Check Python exceptions. |
| Charts do not update | Open browser DevTools -> Console. Check for fetch errors to /stats. |
| Database viewer shows Loading... | Check /api/crossings and /api/quarterly return data. |

## General System Issues

| Symptom | Fix |
| --- | --- |
| ld.so.conf.d errors about DeepStream libs | Ensure /etc/ld.so.conf.d/deepstream.conf contains /opt/nvidia/deepstream/deepstream/lib then sudo ldconfig |
| pyds import errors | pyds is NOT used in current pipeline. Ignore. |
| System slow after boot | Old bike-counter.service may be running. Disable: sudo systemctl stop bike-counter && sudo systemctl disable bike-counter |
| mysql.connector import error | pip3 install mysql-connector-python with export PIP_BREAK_SYSTEM_PACKAGES=1 |

# 12. Build from Scratch

Prerequisites: Fresh Jetson Orin Nano with JetPack 6.x (L4T 39.x), USB camera, internet access, sudo privileges.

## Quick Method (Automated)

cd ~/CounterProject

sudo chmod +x setup.sh

sudo ./setup.sh

## Manual Method (Step by Step)

### Step 1: System Packages

sudo apt-get update && sudo apt-get upgrade -y

sudo apt-get install -y build-essential cmake git wget pkg-config \

python3-dev python3-pip python3-gi python3-gi-cairo \

gir1.2-gstreamer-1.0 gir1.2-gst-rtsp-server-1.0 \

libgstrtspserver-1.0-dev libgstreamer1.0-dev \

libgstreamer-plugins-base1.0-dev \

gstreamer1.0-plugins-bad gstreamer1.0-plugins-good \

gstreamer1.0-plugins-ugly gstreamer1.0-tools \

libcairo2-dev libgirepository1.0-dev

### Step 2: Python Dependencies

export PIP_BREAK_SYSTEM_PACKAGES=1

pip3 install numpy "numpy<2"

pip3 install ultralytics onnx onnxslim

pip3 install fastapi "uvicorn[standard]" jinja2 python-multipart Pillow

pip3 install mysql-connector-python

DO NOT pip install --upgrade pip - the Debian-managed pip must stay intact for dpkg tracking.

### Step 3: Verify DeepStream SDK

ls -la /opt/nvidia/deepstream/deepstream

# If missing, find and symlink:

# sudo mkdir -p /opt/nvidia/deepstream

# sudo ln -sf /path/to/deepstream-X.Y /opt/nvidia/deepstream/deepstream

### Step 4: Build TensorRT Engine (if needed)

The project ships a pre-built yolo11n.engine. Only rebuild if moving to a different GPU:

cd ~/CounterProject

trtexec --onnx=yolo11n.onnx --saveEngine=yolo11n.engine --fp16

### Step 5: MySQL Setup

sudo systemctl start mysql

sudo mysql -u root -e "CREATE DATABASE IF NOT EXISTS bicycle_counter;"

sudo mysql -u root -e "CREATE USER IF NOT EXISTS 'bikecounter'@'127.0.0.1' IDENTIFIED BY 'BikeCount2026!';"

sudo mysql -u root -e "GRANT ALL PRIVILEGES ON bicycle_counter.* TO 'bikecounter'@'127.0.0.1';"

sudo mysql -u root -e "FLUSH PRIVILEGES;"

### Step 6: Verify Installation

cd ~/CounterProject

python3 -c "from ultralytics import YOLO; print('ultralytics OK')"

python3 -c "import cv2; print('OpenCV OK')"

python3 -c "import fastapi; print('FastAPI OK')"

python3 -c "import mysql.connector; print('MySQL connector OK')"

python3 -c "from config_loader import load; print('Config OK:', load()['version'])"

# 13. Possible Upgrades

## Short-term (Feature Additions)

| Upgrade | Description | Effort |
| --- | --- | --- |
| Adjustable parameters via dashboard | Settings panel to change CROSSING_LINE_Y, MIN_CONFIDENCE, CAMERA_BRIGHTNESS without restart | Medium |
| Email/Telegram alerts | Notification when threshold reached (e.g. > 100 bicycles/hour) | Low |
| Historical date picker | View past days data with date navigation in dashboard | Medium |
| Screenshot gallery | Browse captured screenshots in dashboard with thumbnails | Medium |
| WebSocket live events | Replace 500ms polling with WebSocket for real-time updates | Medium |

## Mid-term (Deployment Hardening)

| Upgrade | Description | Effort |
| --- | --- | --- |
| Multi-camera support | Multiple instances with different location_name, unified dashboard | High |
| HTTPS / reverse proxy | nginx with Let's Encrypt TLS for secure remote access | Medium |
| Docker containerization | Package as Docker container with nvidia-docker2 GPU access | High |
| OTA updates | Over-the-air update mechanism for deployed Jetson devices | High |

## Long-term (Advanced Features)

| Upgrade | Description | Effort |
| --- | --- | --- |
| Custom YOLO training | Fine-tune on bicycle-specific dataset for higher accuracy | High |
| Speed estimation | Frame-to-frame displacement with camera calibration | High |
| Multi-class counting | Extend to pedestrians, cars, scooters | Medium |
| Edge-cloud sync | Sync local MySQL to cloud dashboard (Firebase, AWS IoT) | High |
| Analytics API | RESTful API for external systems to query historical data | Medium |
| GPU utilization monitoring | Real-time GPU/CPU/memory stats in dashboard | Low |

# Appendix: Quick Reference Card

## Start / Stop

./run.sh                              # Start manually

sudo systemctl start bike-counter     # Start via systemd

sudo systemctl stop bike-counter      # Stop

## URLs

http://localhost:8080                  # Dashboard

http://localhost:8080/database         # Database viewer

http://localhost:8080/health           # Health check

http://localhost:8080/stats            # Stats JSON

http://localhost:8080/export/csv       # Download 15-min counts CSV

http://localhost:8080/export/detail    # Download all crossings CSV

## Config

nano ~/CounterProject/config.yaml     # Edit config

export BIKE_DASH_PORT=9090            # Override via env var

## Database

mysql -u bikecounter -p'BikeCount2026!' bicycle_counter   # Connect

./backup.sh                           # Backup DB + screenshots

## Logs

tail -f ~/CounterProject/bicycle_counter.log              # Follow log

sudo journalctl -u bike-counter -f                        # Follow systemd log